CHAPTER

NEUROENGINEERING AND BRAIN-COMPUTER INTERFACES: SIGNAL PROCESSING, AI, AND APPLICATIONS

Chapter Overview and Learning Guide

This chapter is designed to guide readers through the evolving landscape of Neuroengineering and Brain-Computer Interfaces (BCIs). Each section builds progressively—from foundational concepts to advanced tools, real-world innovations, signal processing techniques, ethical concerns, and interdisciplinary roles.

To make the chapter both educational and reference-friendly:

• Mini Review boxes provide concise conceptual summaries at the start of each section for quick learning.

• Detailed sections expand on each topic with technologies, techniques, and citations from recent research.

• Researchers and students can dive into technical tools and signal processing methods.

• Designers, ethicists, and interdisciplinary professionals will benefit from insights into usability, human-centered design, and bioethics.

Use this chapter as a gateway to the BCI field, whether you're a beginner seeking structured learning or an expert looking for modern trends and citations.

Table of Contents:

1 Basics of Neuroengineering and Brain-Computer Interface

2 Technological Tools in BCI Research

2.1 Hardware

2.2 Software

3 Signal Processing in Practice

3.1 Filtering, featuring extraction

3.2 Common ML techniques in BCI

4 Recent Innovations and Research Trends

4.1 AI in BCI

4.2 Portable and wearable BCI

5 Human-Centered Design in BCI

5.1 User experience, cognitive load

5.2 Training for patients

6 Ethics and Data Privacy in BCI

6.1 Brain data misuse

6.2 Informed consent and bioethics

7 Interdisciplinary Collaboration

8 Conclusion

1 Basics of Neuroengineering and Brain-Computer Interface

Neuroengineering is a rapidly growing interdisciplinary field that merges engineering methodologies with the study of the nervous system. Its core mission is to understand neural mechanisms and to design technologies that can monitor, restore, or enhance brain function. The scope extends from neural signal processing and prosthetics to brain stimulation and cognitive augmentation.

A central pillar of neuroengineering is the Brain-Computer Interface (BCI)—a communication system that enables users to control external devices using only their brain activity, without relying on peripheral nerves or muscles [1]. BCIs are especially transformative for individuals with motor impairments, offering new pathways for interaction with the environment.

  - What Is a BCI?

A BCI translates neural signals into executable commands by performing the following core tasks:

1 Signal Acquisition – Using non-invasive (EEG), semi-invasive (ECoG), or invasive (microelectrodes) techniques to record brain activity.

2 Preprocessing – Noise removal and artifact filtering to enhance signal quality.

3 Feature Extraction – Identifying patterns such as event-related potentials (ERPs), sensorimotor rhythms (SMRs), or steady-state visually evoked potentials (SSVEPs).

4 Classification/Translation – Machine learning models map features to discrete or continuous control commands.

5 Feedback – The system provides real-time output through devices like screens, wheelchairs, robotic arms, or even speech synthesizers [2][3].

  - Categories of BCI

BCIs can be categorized based on:

• Based on the level of invasiveness, BCIs can be categorized as non-invasive (e.g., EEG), semi-invasive (e.g., ECoG), or invasive (e.g., cortical implants),

• Directionality: Unidirectional (reading from brain) vs. Bidirectional (reading and stimulating),

• Application domain: Assistive (prosthetic control), rehabilitative (stroke therapy), or cognitive (neurofeedback, memory enhancement) [1][4].

  - Key Benefits and Challenges

Benefits:

• Enables communication for locked-in patients (e.g., ALS),

• Enhances neuroplasticity through feedback training,

• Provides real-time control over external systems.

Challenges:

• Low signal-to-noise ratio,

• Long training times for users,

• Ethical concerns over data privacy and brain manipulation,

• Inter-individual variability in brain signals.

  - Interdisciplinary Nature

Neuroengineering and BCI systems represent a multidisciplinary fusion of neuroscience, electrical engineering, biomedical signal processing, computer science, materials science, and ethics. This cross-disciplinary synergy is vital for creating reliable, user-friendly, and ethical BCI systems [5].

- Technological Tools in BCI Research

  - Hardware in Brain-Computer Interface (BCI)

The hardware infrastructure of a BCI system forms the backbone for detecting and interpreting brain activity. It comprises several interdependent components, each tailored for specific applications—ranging from clinical rehabilitation to gaming and neuroprosthetics.

    - Brain Signal Acquisition Units

Electrodes act as the essential link between brain activity and external devices, enabling seamless communication. These include:

•  Non-invasive EEG electrodes (dry or gel-based) for scalp recordings,

• Semi-invasive ECoG arrays are placed on the cortical surface, offering higher spatial resolution than non-invasive methods while minimizing the risks associated with fully invasive implants.

•  Invasive microelectrode arrays are implanted into the cortex for single-neuron resolution.

Each BCI type presents a trade-off among spatial resolution, signal fidelity, and the degree of invasiveness [1].

    - Amplifiers and ADCs

Neural signals, typically in the microvolt range, are highly sensitive to interference and require careful amplification. Low-noise amplifiers boost these signals, followed by ADCs that digitize them for software-based processing. The resolution and sampling rate of analog-to-digital converters (ADCs) play a critical role in determining the quality and usability of downstream neural signals [2].

    - Signal Transmission Modules

Data is transferred to processing units via USB, Bluetooth, or specialized wireless telemetry systems. Emerging platforms use low-power embedded systems for edge computing in portable BCIs [6].

    - Neurostimulation Units

Closed-loop BCIs may include stimulation components like:

• Transcranial Magnetic Stimulation (TMS) for motor cortex modulation,

• tDCS is a non-invasive brain stimulation technique used to enhance cognitive functions by modulating neural activity.

• Deep Brain Stimulation (DBS) in therapeutic BCIs (e.g., Parkinson’s disease).

Integration of sensing and stimulation hardware enables bidirectional BCIs.

    - Wearability and Biocompatibility

Designs now focus on wearable, skin-friendly materials (e.g., flexible electronics, conductive fabrics) to improve comfort, reduce artifacts, and support long-term use outside labs [7].

As hardware evolves, the goal is to enhance signal quality while maximizing user comfort and minimizing invasiveness, enabling scalable, real-world BCI applications.

  - Software in BCI

While hardware collects raw brain signals, the software stack in a Brain-Computer Interface (BCI) system is responsible for interpreting those signals, learning user-specific patterns, and providing actionable output in real-time. A well-designed software architecture is crucial for transforming neural data into meaningful interactions.

    - Signal Processing and Analysis Frameworks

Several open-source and proprietary platforms are available for neurodata analysis:

• EEGLAB (MATLAB-based): Popular for offline EEG analysis and ICA decomposition [8].

• MNE-Python is a versatile toolkit in Python used for processing, visualizing, and analyzing electrophysiological signals such as EEG, MEG, and ECoG.

• BCILAB: Extends EEGLAB for online and offline BCI experimentation.

• OpenViBE: Designed for real-time BCI development, supports acquisition, signal processing, and user feedback.

These frameworks offer tools for preprocessing (e.g., band-pass filtering, artifact rejection), feature extraction (e.g., spectral power, ERP components), and classification.

    - Machine Learning Libraries

Modern BCIs rely on machine learning to detect user intention. Some popular libraries used in BCI software include:

• Scikit-learn – Offers classical ML models like SVM, LDA, and k-NN.

• TensorFlow & PyTorch – Widely used for deep learning models like CNNs and RNNs in EEG-based decoding tasks.

• xDAWN and CSP – Feature extraction algorithms used as part of ML pipelines in tools like pyRiemann or MOABB [9].

    - Real-Time Control and Integration

BCI software must facilitate low-latency communication with external devices to enable real-time feedback and control in closed-loop systems.

• BCI2000 – A general-purpose software framework supporting many BCI paradigms.

• LabStreamingLayer (LSL) – Synchronizes data from multiple sources in real-time.

• ROS (Robot Operating System) – Used to connect BCI outputs with robotic or IoT systems.

Custom Graphical User Interfaces (GUIs) or dashboards often provide real-time feedback, crucial for user learning and interaction.

    - Simulation and Testing Tools

Simulations using virtual environments or synthetic data generators are used to:

• Test algorithms without human input,

• Simulate neurofeedback training protocols,

• Benchmark performance across users and setups.

Frameworks like BCI Competition Datasets, NeuroPype, and NeuroSim are used for reproducible research and comparison [10].

- Signal Processing in Practice

  - Filtering and Feature Extraction

Brain signals recorded via EEG or invasive methods are often contaminated with noise from muscle activity, eye movements (EOG), power line interference, and environmental artifacts. The initial step in signal processing involves applying appropriate filtering techniques to enhance the quality of neural signals.

    - Filtering Techniques

• Band-pass Filtering: EEG frequencies of interest typically lie between 0.5 and 70 Hz. Band-pass filters isolate specific neural frequency bands commonly associated with brain activity, such as:

  - Delta (0.5–4 Hz)

  - Theta (4–8 Hz)

  - Alpha (8–13 Hz)

  - Beta (13–30 Hz)

  - Gamma (>30 Hz)

Finite Impulse Response (FIR) and Infinite Impulse Response (IIR) filters are widely utilized for their accuracy and reliable performance in signal processing applications[11].

• Artifact Removal: Techniques like Independent Component Analysis (ICA) and regression methods remove ocular and muscular artifacts, improving signal clarity for subsequent analysis [12].

    - Feature Extraction Approaches

Extracting features reduces high-dimensional raw data to key parameters that represent neural states:

• Time-Domain Features: Statistical measures such as mean, variance, root mean square, and Hjorth parameters quantify temporal signal characteristics.

• Frequency-Domain Features: Power spectral density (PSD) estimation via Fourier Transform or Welch’s method captures frequency content, critical for detecting rhythms like sensorimotor rhythms (SMR) used in motor imagery BCI.

• Spatial Filters: Common Spatial Patterns (CSP) enhance discriminability by emphasizing spatial differences in EEG signals linked to different mental tasks [13].

• Time-Frequency Analysis: Wavelet transforms allow simultaneous time and frequency resolution, capturing transient brain dynamics [14].

    - Importance for BCI

Efficient filtering and feature extraction minimize computational load and improve classification accuracy, vital for real-time BCI feedback. Flexible feature extraction methods facilitate individualized calibration of Brain-Computer Interfaces (BCIs) for different users.

  - Common Machine Learning Techniques in BCI

The integration of machine learning in BCI systems has dramatically improved the accuracy and robustness of decoding neural signals. Key ML approaches include:

    - Traditional Machine Learning Algorithms

• Support Vector Machines (SVM): Effective for binary classification of EEG features, SVM finds the optimal hyperplane separating classes in high-dimensional space. Its kernel functions handle non-linear separations [11].

• Linear Discriminant Analysis (LDA) is an efficient classification technique that reduces dimensionality while enhancing the distinction between different classes. Widely used due to low computational cost and good performance on small datasets [13].

• Random Forests and k-Nearest Neighbors (k-NN): Ensemble and instance-based classifiers used for multi-class classification, often as benchmarks or part of hybrid models.

    - Deep Learning Approaches

• Convolutional Neural Networks (CNNs): CNNs automatically extract hierarchical spatial and temporal features from EEG data, reducing the need for manual feature engineering. They excel in recognizing complex patterns, especially in motor imagery tasks [11].

• Recurrent Neural Networks (RNNs) and Long Short-Term Memory (LSTM) networks effectively capture temporal dependencies in EEG signals, making them valuable for continuous BCI tasks like speech or movement decoding [15].

• Hybrid Models: Combining CNNs with RNNs or integrating attention mechanisms further enhances performance and adaptability.

    - Transfer Learning and Domain Adaptation

A major challenge in BCI research is the variability in neural signals both across different individuals and across multiple recording sessions. Transfer learning methods leverage pre-trained models and adapt them to new users with minimal additional data, improving generalization and reducing calibration time [16].

    - Challenges and Considerations

• Overfitting on limited data

• Real-time computational constraints

• Interpretability of deep models

• Need for large, diverse datasets

- Recent Innovations and Research Trends

  - AI in BCI

Artificial Intelligence (AI) is integral to advancing brain-computer interfaces, transforming raw neural data into actionable commands with unprecedented precision and adaptability. AI plays a multifaceted role in BCI, including signal decoding, adaptive learning, noise reduction, user intent prediction, and enabling real-time interaction.

    - Deep Learning for Feature Extraction and Classification

Deep neural networks, especially CNNs and transformers, automatically learn discriminative features from raw EEG or invasive neural recordings. This reduces the dependency on handcrafted features, enabling end-to-end training and improving accuracy for complex tasks like imagined speech or motor control [11][17].

    - Reinforcement Learning for Adaptive Control

Reinforcement learning (RL) algorithms enable BCI systems to adaptively improve through trial and error, optimizing feedback and control policies in real-time. RL enhances closed-loop BCI systems by allowing continuous calibration and personalization without explicit recalibration sessions [18].

    - Hybrid AI Models and Multimodal Integration

Combining AI methods, such as integrating deep learning with probabilistic models or combining EEG with other biosignals (EMG, EOG), results in robust hybrid BCIs. These systems improve signal reliability and context awareness, expanding BCI usability in real-world environments [19].

    - Personalization and Transfer Learning

AI facilitates personalized BCI by adapting models to individual brain patterns and conditions using transfer learning and domain adaptation techniques, reducing training time and enhancing cross-session stability [16].

    - Challenges

• High computational demand

• Need for large labeled datasets

• Interpretability of AI decisions

• Ethical concerns around autonomous adaptation

  - Portable and Wearable BCI

Portable and wearable brain-computer interfaces represent a critical evolution from bulky, stationary lab setups to lightweight, user-friendly devices capable of real-time neural monitoring in everyday environments.

    - Hardware Innovations

Recent progress in dry and semi-dry electrodes eliminates the need for conductive gels, enhancing comfort and ease of use. Flexible and textile-based electrodes integrated into headwear or caps improve wearability. Compact, energy-efficient amplifiers, analog front-end circuits, and embedded processors enable real-time signal acquisition and processing in portable Brain-Computer Interface (BCI) systems[20][6].

    - Wireless Data Transmission and Edge Computing

Wireless protocols like Bluetooth Low Energy (BLE) and ultra-wideband (UWB) facilitate untethered data streaming. Coupled with edge computing, these devices perform preliminary signal processing locally, reducing latency and dependence on cloud infrastructure while preserving user privacy [21].

    - Application Domains

• Neurorehabilitation: Wearable BCIs support stroke and spinal cord injury recovery by enabling real-time feedback and motor intention decoding during therapy [22].

• Health Monitoring: Continuous monitoring of sleep, stress, or cognitive states for mental health assessment.

• AR and VR technologies improve immersive interactions by integrating direct neural inputs, creating more responsive and engaging environments.

• Assistive technologies enhance communication and control capabilities for individuals with disabilities, enabling greater independence and interaction through BCIs.

    - Challenges

• Motion artifacts and environmental interference can significantly reduce signal clarity, affecting overall data quality.

• Limited battery life and need for energy-efficient designs.

• Balancing device comfort, aesthetics, and usability.

• Data security and privacy concerns in portable use.

- Human-Centered Design in BCI

  - User Experience and Cognitive Load in BCI

Effective brain-computer interfaces depend not only on technical accuracy but also on the user’s comfort and cognitive capacity to operate them seamlessly. User experience (UX) design in BCI aims to create intuitive, accessible, and engaging systems that reduce mental strain and improve sustained use.

    - Understanding Cognitive Load

Cognitive load represents the overall mental effort required to interpret and react to stimuli while using a Brain-Computer Interface (BCI). High cognitive load can cause rapid fatigue, reduce accuracy, and impair user motivation. BCI tasks demanding sustained attention, complex mental imagery, or frequent recalibration increase this load [23].

    - Strategies to Reduce Cognitive Load

• Adaptive Interfaces: Systems that adjust task difficulty and feedback intensity based on real-time performance reduce unnecessary strain.

• Multimodal Feedback: Combining visual, auditory, and haptic cues enhances understanding and reduces the need for intense concentration.

• Simplified Control Paradigms: Using fewer control commands or naturalistic neural signals (e.g., motor imagery) lowers user effort.

• Personalization: Customizing the BCI experience based on a user's cognitive abilities and preferences enhances both usability and effectiveness. [24].

    - Measuring User Experience

Objective metrics (e.g., EEG-based workload indicators, performance accuracy) and subjective assessments (e.g., questionnaires on comfort and fatigue) guide iterative improvements in BCI design [25].

    - Importance for Clinical and Consumer BCIs

Optimizing UX and cognitive load is vital for clinical populations with limited endurance and for consumer applications requiring long-term engagement. Enhanced usability directly correlates with adoption rates and functional benefits.

  - Training for Patients

Training is a critical component in patient-centered BCI applications, particularly for those with motor impairments or neurological disorders. Successful BCI use often requires learning to modulate neural activity deliberately, which demands structured, adaptive training programs.

    - Neuroplasticity and Learning

Repeated BCI use fosters neuroplastic changes—functional and structural brain adaptations—that enhance control accuracy and functional recovery. Training protocols capitalize on this plasticity to reinforce neural pathways and restore lost functions [26].

    - Training Protocols and Strategies

• Progressive Task Difficulty: Starting with simple, low-demand tasks and gradually increasing complexity helps patients build confidence and skill.

• Real-Time Feedback: Visual, auditory, or haptic feedback aids users in understanding the impact of their neural signals, facilitating learning.

• Hybrid Systems: Combining BCIs with physical therapy or robotic assistance accelerates rehabilitation outcomes by engaging multiple modalities [27].

• Motivation and Engagement: Gamified and goal-oriented training keeps patients motivated and improves compliance.

    - Challenges and Considerations

• Variability in patients’ cognitive and physical capacities requires personalized training.

• Fatigue and frustration can reduce training effectiveness; balancing intensity and rest is crucial.

• Continuous monitoring and adaptive adjustment of training protocols are essential to ensure optimal user progress and performance in BCI applications.

    - Clinical Outcomes

Studies show that structured BCI training improves motor recovery, communication abilities, and quality of life in patients with stroke, ALS, spinal cord injury, and other conditions [27].

- Ethics and Data Privacy in BCI

  - Brain Data Misuse

Brain-computer interfaces capture highly sensitive neural signals that potentially expose users’ private cognitive states. This raises significant risks if brain data is mishandled, stolen, or misused by malicious actors or even by organizations without proper oversight.

    - Nature of Brain Data

Brain data may include:

• Cognitive states (attention, fatigue, intentions),

• Emotional responses,

• Mental health indicators,

• Personal preferences and thoughts [29].

Unlike other biometric data, brain data is uniquely intimate and inherently challenging to fully anonymize.

    - Risks of Misuse

• Privacy Violations: Unauthorized data access can reveal private mental information.

• Manipulation and Coercion: Neural data might be exploited to influence decisions or behavior.

• Discrimination: Employers or insurers could misuse brain data to discriminate based on cognitive or emotional traits.

• Security Threats: Brain data interception or hacking can threaten user safety and autonomy [30].

    - Regulatory and Technical Challenges

Current regulations on data protection frequently lack specific provisions for safeguarding brain-related information. Defining ownership, consent, and permissible use requires new legal frameworks. Technological safeguards include:

• Data encryption,

• Secure storage,

• Anonymization techniques,

• User-controlled access permissions [31].

    - Ethical Imperatives

BCI developers and policymakers must prioritize user autonomy, transparency, and strict data governance to prevent misuse and build public trust.

  - Informed Consent and Bioethics

The ethical deployment of brain-computer interfaces demands rigorous attention to informed consent and broader bioethical principles to protect participants and users.

    - Importance of Informed Consent

Participants must receive clear, comprehensive information about:

• The nature and purpose of BCI intervention,

• Potential risks and benefits,

• Data collection, storage, and sharing policies,

• Right to withdraw without penalty [29].

This is especially crucial for vulnerable populations such as patients with cognitive impairments or severe disabilities.

    - Bioethical Principles in BCI

• Autonomy: Respecting users’ control over their bodies and data.

• Beneficence: Maximizing benefits and minimizing harm.

• Non-maleficence: Avoiding physical or psychological harm.

• Justice: Ensuring fair access and avoiding discrimination [32].

    - Challenges and Considerations

• Brain-Computer Interfaces (BCIs) have the potential to cause unforeseen psychological impacts or lead to user reliance over time.

• Long-term impacts remain under-researched, raising concerns about safety.

• Cultural, social, and individual differences affect consent comprehension and ethical considerations.

• Maintaining ethical safeguards while fostering innovation is challenging yet essential for responsible technological progress.

    - Toward Ethical Standards and Guidelines

Developing international consensus guidelines and multidisciplinary oversight is key to maintaining ethical standards, protecting human rights, and fostering public trust in BCI technologies [33].

- Interdisciplinary Collaboration

Brain-computer interfaces exemplify a deeply interdisciplinary field where progress depends on the seamless integration of multiple domains.

  - Diverse Disciplines Involved

• Neuroscience: Understanding brain function and neural signal generation.

• Electrical & Computer Engineering: Designing hardware for signal acquisition and processing.

• Computer Science & AI: Developing algorithms, machine learning models, and software for decoding brain signals.

• Psychology & Cognitive Science: Studying user behavior, cognitive load, and training protocols.

• Medicine & Rehabilitation: Applying BCIs for patient care and therapeutic interventions.

• Ethics & Law: Ensuring responsible development, data privacy, and regulatory compliance.[34]

  - Benefits of Collaboration

• Accelerates innovation by leveraging varied expertise.

• Enables comprehensive solutions that address technical, clinical, and ethical dimensions.

• Fosters user-centered design through psychological and clinical insights.

• Supports robust regulatory and ethical frameworks informed by legal scholars.

  - Challenges and Opportunities

• Effective communication across disciplines requires shared terminology and collaborative culture.

• Integration of diverse methodologies and standards can be complex.

• Sustained funding and institutional support are crucial for fostering effective interdisciplinary collaboration in BCI research.

• Emerging fields like neuroethics highlight new collaboration needs between scientists and philosophers.[35][36]

  - Future Directions

Strengthening interdisciplinary education, joint research centers, and public-private partnerships will be pivotal in advancing BCI technologies that are innovative, ethical, and impactful.

- Conclusion

Neuroengineering and brain-computer interfaces represent a transformative frontier where technology meets the human mind. The continuous evolution of hardware tools, sophisticated signal processing techniques, and AI-driven innovations has expanded BCI capabilities beyond clinical applications to consumer and research domains.

While promising, these advancements bring critical ethical and privacy challenges that must be addressed through informed consent, responsible data handling, and interdisciplinary oversight.

The future of BCI depends on fostering collaboration across neuroscience, engineering, computer science, psychology, medicine, and ethics to design systems that are effective, accessible, and human-centered.

By embracing this holistic approach, the potential of BCIs to restore, augment, and revolutionize human capabilities can be responsibly realized—ushering in a new era of brain-machine symbiosis with profound societal impact.

- References

[1] Lebedev, M. A., & Nicolelis, M. A. L. (2006). Brain–machine interfaces: past, present and future. Trends in Neurosciences, 29(9), 536–546. https://doi.org/10.1016/j.tins.2006.07.004

[2] He, B. (Ed.). (2013). Neural Engineering. Springer.
[3] Wolpaw, J. R., & Wolpaw, E. W. (Eds.). (2012). Brain-Computer Interfaces: Principles and Practice. Oxford University Press.
[4] Nicolas-Alonso, L. F., & Gomez-Gil, J. (2012). Brain-computer interfaces: a review. Sensors, 12(2), 1211–1279. https://doi.org/10.3390/s120201211

[5] Bashashati, A., Fatourechi, M., Ward, R. K., & Birch, G. E. (2007). A survey of signal processing algorithms in brain-computer interfaces based on electrical brain signals. Journal of Neural Engineering, 4(2), R32–R57. https://doi.org/10.1088/1741-2560/4/2/R03

[6] Casson, A. J. (2019). Wearable EEG and beyond. Biomedical Engineering Letters, 9(1), 53–71. https://doi.org/10.1007/s13534-018-00093-x

[7] Ferrari, L. M., et al. (2021). Soft, stretchy, fully implantable bioelectronics for closed-loop neuromodulation. Science Advances, 7(11), eaba3517. https://doi.org/10.1126/sciadv.aba3517

[8] Delorme, A., & Makeig, S. (2004). EEGLAB: an open-source toolbox for analysis of single-trial EEG dynamics. Journal of Neuroscience Methods, 134(1), 9–21. https://doi.org/10.1016/j.jneumeth.2003.10.009

[9] Barachant, A., Bonnet, S., Congedo, M., & Jutten, C. (2010). Common Spatial Pattern revisited by Riemannian geometry. 2010 IEEE International Workshop on Multimedia Signal Processing, 472–476. https://doi.org/10.1109/MMSP.2010.5661988

[10] Tangermann, M., Müller, K.-R., Aertsen, A., Birbaumer, N., Braun, C., Brunner, C., ... & Kübler, A. (2012). Review of the BCI competition IV. Frontiers in Neuroscience, 6, 55. https://doi.org/10.3389/fnins.2012.00055

[11] Roy, Y., Banville, H., Albuquerque, I., Gramfort, A., Falk, T. H., & Faubert, J. (2020). Deep learning-based electroencephalography analysis: a systematic review. Journal of Neural Engineering, 17(4), 041001. https://doi.org/10.1088/1741-2552/ab260c

[12] Urigüen, J. A., & Garcia-Zapirain, B. (2015). EEG artifact removal—state-of-the-art and guidelines. Journal of Neural Engineering, 12(3), 031001. https://doi.org/10.1088/1741-2560/12/3/031001

[13] Lotte, F., Congedo, M., Lécuyer, A., Lamarche, F., & Arnaldi, B. (2018). A review of classification algorithms for EEG-based brain–computer interfaces: a 10 year update. Journal of Neural Engineering, 15(3), 031005. https://doi.org/10.1088/1741-2552/aab2f2

[14] Subasi, A. (2021). EEG signal classification using wavelet feature extraction and a mixture of expert model. Expert Systems with Applications, 168, 114302. https://doi.org/10.1016/j.eswa.2020.114302
[15] Zhang, X., et al. (2021). Deep learning in EEG-based brain-computer interfaces: a review. Journal of Neural Engineering, 18(4), 041002. https://doi.org/10.1088/1741-2552/ac0d36

[16] Kwon, D., & Kim, D. (2023). Transfer learning approaches for brain-computer interfaces: A comprehensive review. Frontiers in Neuroscience, 17, 1100302. https://doi.org/10.3389/fnins.2023.1100302

[17] Lawhern, V. J., et al. (2018). EEGNet: a compact convolutional neural network for EEG-based brain–computer interfaces. Journal of Neural Engineering, 15(5), 056013. https://doi.org/10.1088/1741-2552/aace8c

[18] Li, Y., & Shi, J. (2022). Reinforcement learning for brain-computer interfaces: a review. IEEE Transactions on Neural Systems and Rehabilitation Engineering, 30, 213–224. https://doi.org/10.1109/TNSRE.2022.3149241

[19] Lotte, F., & Roy, R. N. (2021). Multimodal brain-computer interfaces: principles, challenges and opportunities. Frontiers in Neuroscience, 15, 650161. https://doi.org/10.3389/fnins.2021.650161

[20] Chi, Y. M., Wang, Y. T., Wang, Y., Maier, C., Jung, T. P., & Cauwenberghs, G. (2012). Dry and noncontact EEG sensors for mobile brain–computer interfaces. IEEE Transactions on Neural Systems and Rehabilitation Engineering, 20(2), 228–235. https://doi.org/10.1109/TNSRE.2011.2179299

[21] Ghafoor, K. Z., et al. (2021). Edge computing for real-time EEG signal processing in brain-computer interface applications. IEEE Access, 9, 121289–121302. https://doi.org/10.1109/ACCESS.2021.3107196

[22] Coyle, D., Ward, T., & McDonough, S. (2020). Wearable brain-computer interfaces for neurorehabilitation: current status and future prospects. Frontiers in Neuroscience, 14, 575. https://doi.org/10.3389/fnins.2020.00575

[23] Gramann, K., et al. (2017). Cognition in action: Imaging brain/body dynamics in mobile humans. NeuroImage, 159, 18–30. https://doi.org/10.1016/j.neuroimage.2017.07.016

[24] Lotte, F., & Jeunet, C. (2018). Towards improved BCI usability: cognitive load and user experience. Proceedings of the IEEE, 106(6), 1029–1042. https://doi.org/10.1109/JPROC.2018.2812042

[25] Zander, T. O., & Kothe, C. (2011). Towards passive brain–computer interfaces: applying brain–computer interface technology to human–machine systems in general. Journal of Neural Engineering, 8(2), 025005. https://doi.org/10.1088/1741-2560/8/2/025005

[26] Coyle, D., Ward, T., & Markham, C. (2012). Brain–computer interface training for neurorehabilitation: a review. Brain Sciences, 2(3), 325–356. https://doi.org/10.3390/brainsci2030325

[27] Ang, K. K., Guan, C., Phua, K. S., & Kuah, C. W. K. (2014). Brain-computer interface-based robotic end effector system for wrist and hand rehabilitation: results of a three-armed randomized controlled trial for chronic stroke. Frontiers in Neuroengineering, 7, 30. https://doi.org/10.3389/fneng.2014.00030

[28] Soekadar, S. R., Birbaumer, N., Slutzky, M. W., & Cohen, L. G. (2015). Brain-machine interfaces in neurorehabilitation of stroke. Neurobiology of Disease, 83, 536–544. https://doi.org/10.1016/j.nbd.2014.11.025

[29] Nijboer, F., Clausen, J., Allison, B. Z., & Haselager, P. (2013). The Asilomar Survey: Stakeholders’ Opinions on Ethical Issues Related to Brain-Computer Interfacing. Neuroethics, 6(3), 541–578. https://doi.org/10.1007/s12152-012-9167-1

[30] Ienca, M., & Andorno, R. (2017). Towards new human rights in the age of neuroscience and neurotechnology. Life Sciences, Society and Policy, 13(1), 5. https://doi.org/10.1186/s40504-017-0050-1

[31] Nijboer, F., & Clausen, J. (2020). Ethics of Brain-Computer Interfaces: A Systematic Review. In Brain-Computer Interfaces (pp. 171–188). Springer. https://doi.org/10.1007/978-3-030-58796-3_9

[32] Moreno, J. D., & Farah, M. J. (2015). Ethics of Neuromodulation: Brain Stimulation in Psychiatric and Neurologic Disorders. Current Opinion in Psychiatry, 28(6), 522–526. https://doi.org/10.1097/YCO.0000000000000190

[33] Ienca, M., Haselager, P., & Emanuel, E. J. (2018). Brain leaks and consumer neurotechnology. Nature Biotechnology, 36(9), 805–810. https://doi.org/10.1038/nbt.4231

[34] Wolpaw, J. R., & Wolpaw, E. W. (Eds.). (2012). Brain-Computer Interfaces: Principles and Practice. Oxford University Press.
[35] Nijholt, A. (2019). Brain-Computer Interfaces for Artistic Expression: A Promising Challenge. Brain Sciences, 9(10), 285. https://doi.org/10.3390/brainsci9100285

[36] Ienca, M., & Haselager, P. (2016). Hacking the brain: Brain-computer interfacing technology and the ethics of neurosecurity. Ethics and Information Technology, 18(2), 117–129. https://doi.org/10.1007/s10676-016-9397-2

- Abbreviations

| Abbreviation | Full Form |
| --- | --- |
| BCI | Brain-Computer Interface |
| EEG | Electroencephalography |
| ECoG | Electrocorticography |
| fMRI | Functional Magnetic Resonance Imaging |
| fNIRS | Functional Near-Infrared Spectroscopy |
| MRI | Magnetic Resonance Imaging |
| MEG | Magnetoencephalography |
| LFP | Local Field Potential |
| AI | Artificial Intelligence |
| ML | Machine Learning |
| CNN | Convolutional Neural Network |
| SVM | Support Vector Machine |
| ICA | Independent Component Analysis |
| CSP | Common Spatial Patterns |
| UX | User Experience |
| HCI | Human-Computer Interaction |
| EMG | Electromyography |
| IoT | Internet of Things |
| BMI | Brain-Machine Interface |
| API | Application Programming Interface |
| ERP | Event-Related Potential |
| TMS | Transcranial Magnetic Stimulation |
| DBS | Deep Brain Stimulation |
| HMI | Human-Machine Interface |
| UI | User Interface |
| VR | Virtual Reality |
| AR | Augmented Reality |
| GDPR | General Data Protection Regulation |